Project: Investigating Haptic Feedback in Teleoperation

| Date: 04/09/2026 |  |
| --- | --- |
| Student Engineer: | Joseph Schneider |
| Student ID: | N11973463 |
| Supervisor: | Professor Will Browne |

Executive Summary

Current teleoperation practices use visual-only feedback, which can lead to damage to the robot and its environment if teleoperation is not performed carefully, as inexperienced operators can accidentally output too much force and damage delicate objects (Wichmann et al. (2014)).

Multimodal feedback is often required for full performance efficiency (Li et al. (2022)), however implementing real-time multimodal feedback is difficult due to signal latency that increases as the amount of feedback transmitted increases. All forms of feedback (haptic, audio, visual [RGB, LIDAR, IR]) require minimal signal latency (1-50ms) for peak performance and high signal fidelity for peak immersion, which is very difficult to implement simultaneously for every modality, as doing so requires significant information bandwidth in a signal path with limited throughput (Wichmann et al. (2014)).

Another problem is implementing and visualising real-time LIDAR data in VR with minimal latency, as most SLAM algorithms are very computationally heavy and struggle to perform at frame-rates comfortable for human operators (>20Hz) while maintaining high fidelity and accuracy (Kuo et al. (2021)).

The hypothesised solution to these problems is implementing limited multimodal feedback (haptic, audio, visual [RGB, LIDAR, IR]) into a robot able to perform dextrous, bimanual manipulation via VR teleoperation, and iterating through combinations of feedback levels (e.g., multimodal combinations, bandwidth combinations (fidelity of individual modalities, particularly LIDAR), etc.) to determine the most effective feedback methods for teleoperation. The hypotheses being tested are that multimodal teleoperation is more efficient at completing manipulation tasks than visual-only teleoperation, and that adding new feedback modes into visual-only teleoperation does not decrease an operator’s ability to complete a task.

Using haptic force/pressure feedback would allow operators to use appropriate levels of force to manipulate delicate objects, significantly decreasing risk of damages (Zhou et al. (2023)). Combined with different modes of visual feedback, it would allow operation in situations where RGB-only visual data is unhelpful (e.g., finding a light switch in a dark room, manipulating occluded objects).

For users, it could also increase the speed/efficiency of general manipulation tasks that a helper robot would encounter, saving time and effort for both the operator and the human being helped. For industry, this system could be implemented in most situations requiring dextrous manipulation, allowing for multi-disciplinary use and eliminating the need for multiple specialised systems (Shukla et al. (2016)).

General Objective

This project aims to implement different feedback types (haptic, audio, RGB, LIDAR, IR) into multimodal VR teleoperation and evaluate their effects on operator performance and behaviour. Most current teleoperation practices implement visual-only feedback, meaning a full understanding of the robot’s situation is rarely provided to the operator, which can lead to inefficiency as well as damage to the robot and its surroundings (Li et al. (2022)). The main goal of this project is to deliver a dextrous, bimanual system that uses multimodal feedback to implement real-time VR teleoperation with custom haptic feedback gloves and evaluate different combinations of feedback for their effect on the operator’s performance and behaviour, in the context of a general helper robot being teleoperated at the Queensland Centre for Robotics (S11 at QUT).

While previous research has been done into multimodal robotics at QUT (Zhang et al. (2026)), it did not investigate VR teleoperation with haptic force feedback, different visual modes or the difficulties of implementing real-time multimodal feedback that result from signal latency that increases as the amount of feedback transmitted increases. Some difficulties that arise as a result of restricted feedback throughput are low frame rate, low signal quality and signal delays that cause broken immersion and significantly worse operator performance. This is a research gap that led to the research question:

How does the availability of audio and haptic force feedback affect human performance and behaviour during teleoperation of robots with varied visual data?

Key Finding from the Literature - Impact on Your Project Design

“Teleoperation’s primary reliance on visual feedback limits the operator’s efficiency in situations with stiff contact or poor visibility” (Gong et al. (2024)). Teleoperation using visual-only feedback is not immersive and is often risky due to forces not being correctly monitored and regulated. Adding a different feedback mode into visual-only teleoperation universally increases operator performance and confidence in completing tasks (Ohno et al. (2026)). Implementing either haptic or audio feedback decreases task time, task errors, equipment damage, maximum force exerted and mental load/fatigue on the operator, while increasing operator trust and confidence.

Inversely, it was also found that the performance improvement depends on factors like if the added information feedback is within computational and bandwidth limits, as factors like signal latency can significantly decrease performance efficiency (see Appendix A). This implies that a certain amount of information feedback can be detrimental to teleoperation performance, leading to the two hypothesis that will be investigated in the project:

- Multimodal teleoperation is more efficient at completing manipulation tasks than visual-only teleoperation.

- Adding new feedback modes (haptic, audio, visual [LIDAR, IR]) into visual-only teleoperation does not decrease an operator’s ability to complete a task.

Stakeholders & Resources

There are several possible end users for this project, but the main end user is industries looking for generalised, mobile teleoperation systems that can be implemented in most environments (e.g., low light, hazardous to humans, etc.) and are able to moderate force and thus lessen damage to fragile items (Shukla et al. (2016)). Consultation to determine specifics of potential industry usage in order to inform the project is available at ARM Hub. Since EGH490 is an individual project, there are no team members that need to be consulted, however the supervisor of the project (Professor Will Browne) does need to be informed about the project’s progress and outcomes, and is available for consultation if required.

There are several resources that will be used to complete this project. The hardware resources will be the lab and floor space of S11, the Unitree G1 equipped with the RH56DFX Inspire Hand, customised gloves that provide information input and haptic feedback (LucidVR. (2026)) and a VR headset that provides visual and auditory feedback. The software resources will be GitHub for source control, Unitree’s GitHub repositories (particularly Unitree’s xr_teleoperate (Unitreerobotics. (2026)) for real time teleoperation) and official documentation, ROS2 for visualising the SLAM data and manually controlling the G1’s joint movements, and the LucidGlove GitHub repository for the haptic glove firmware (LucidVR. (2026)).

For testing, google forms will be used to collect survey data. For ethics and compliance, public approved ethics applications from QUT will be analysed, as they will be the most accurate and compliant to QUT’s ethics regarding testing robotics with human volunteers.

Project Methodology

The project methodology will follow an iterative engineering design based on the V-shaped project model. The V-shaped project model is a mostly linear structured model made of two sections, the design stage and validation stage, each with steps that correspond to another in the other stage (see Appendix B, Figure 1).

After the design section has been completed, the results of each validation step get compared to the intended outcome from the corresponding design step, and if it meets the criteria it passes to the next stage of validation. If it doesn’t meet the criteria (e.g., glove finger movement does not cause equal robot finger movement in the subsystem test) then the process is iterated upon, starting again from the design step that failed the criteria. The specifics of the V-shaped project model used in this project are as follows:

# Design

## System Concept through Verification Study

Study current literature on the state of teleoperation practices and use it to create a literature review. Use the literature review to find research gaps to investigate (identified research gap: teleoperation with more than two feedback modalities).

## Requirements Analysis

Research design requirements for multimodal teleoperation (e.g., maximum signal latency before feedback becomes unreliable, minimum processing power to run real-time SLAM algorithms, force feedback limits set by ISO/TS 15066 (ISO. (2016)), etc.) and investigate resources available for use in the project (e.g., investigating the processing speed of the Unitree G1 to see if it can handle SLAM and VR feedback simultaneously, investigating the exact model of the robot hands available and whether they contain pressure sensors, etc.).

## System Specification

Create a high-level design of the system that meets the requirements identified in the previous step. Some examples of high-level design choices include using the Unitree G1 to meet the SLAM and bilateral manipulation requirements, using the RH56DFX Inspire Hands to meet the dextrous manipulation and haptic sensors requirements, and using the LucidGlove framework to meet the haptic force feedback and budget requirements.

## Component Design

Use the LucidGlove framework (LucidVR. (2026)) to design and fabricate the components of the haptic feedback gloves in accordance with the requirements identified (e.g., accuracy of measurements, maximum force feedback allowed, etc.). Additional alteration must be done in order to generalise the fit of the glove as to not introduce confounding variables (e.g., restricted hand mobility or glove slippage) during testing with volunteers.

## Module Design

Design and implement all feedback modes individually (In the order: haptic, RGB vision, audio, LIDAR vision and IR vision) before integrating them in combination with others after testing so that any inherent issues can be identified and mitigated early. This step will be the most iterated upon due to the challenges of integrating so many subsystems into one system (particularly LIDAR), as thus will be given a large timeline to allow for integration issues to be mitigated.

# Validation

## Module/Subsystem Test

Test components and individual feedback methods, measure quality of performance using refresh rate, signal delay and feedback quality (individual to modality, e.g., image resolution for RGB). If the quality of the feedback does not meet the requirements identified in the design stage, return to the module design step and iterate a solution (e.g., if high fidelity SLAM takes too much processing power, lower fidelity and test again), and if the quality of the component does not meet the requirements, then return to the component design step.

## Integration Test

Test the integration of multimodal combinations to validate feedback synchronisation and real-time performance. Measure quality of integration with data bandwidth, refresh rate, signal delay, feedback synchronisation, computational load and consistency in results.

## Acceptance Test

Verify that each of the high-level requirements the system was designed to fit have been met through individually testing them. Requirements will be split into soft constraints and hard constraints. Hard constraints will be related to core functionality and safety, for example constraining force feedback to under 5N (ISO. (2016)), real time operation must be possible, all core feedback modalities are functional, etc. Soft constraints have flexible limits, for example low latency (~<200ms), high update rate (~>10Hz), low bandwidth, etc. Once the requirements have been met, the system has fulfilled the goal of implementing real time teleoperation with multimodal feedback.

## Experimental Test

Begin testing with 10-15 volunteers, because testing with so many conditions will make the tests long (possible over an hour) and testing with more than 15 participants could mean over 15 hours of tests. This would negatively affect the project timeline, so 15 participants will be the limit. Testing for quantitative data will consist of pick-and-place tasks under various conditions (randomised as to not let sequences create familiarity with specific modalities and thus increase the volunteer’s performance in only those tests).

Conditions will be split into “modality” and “task” conditions, where modality conditions are random combinations of each feedback modality and task conditions are variations to the pick-and-place task. Some task conditions will be the object’s softness (solid (wooden cube) vs malleable (purple dinosaur plushie)), noise level (silent (wooden cube) vs makes noise during interaction (empty plastic water bottle)), colour and if the object is occluded.

Testing for qualitative data will consist of one post-task survey per feedback combination as to not to overwhelm the participants with questions, and a different survey at the end of testing. The post-task surveys will measure perceived mental load using the NASA Task Load Index, and the final survey will measure perceived usability of the system as a whole using the System Useability Scale.

## Operation

Analyse and evaluate results from previous step, determine whether the null hypothesis has been rejected or not, use analysis to answer research question. Present final outcome of capstone project (system, results and analysis) to Professor Will Browne and other examiners through project delivery report and oral defence of thesis.

Deliverables

The end goal of this project is to deliver a system capable of real-time multimodal teleoperation, and to present it along with a final project delivery report for a final oral defence presentation, as well as delivering a working demonstration. All code, designs and testing results will also be delivered. Additional interim progress documentation will also be delivered, consisting of this document and reviews to it, initial conceptual designs and later detailed designs, documentation of prototype stages, an interim progress report before the final, and weekly progress meetings with the project supervisor. Depending on time constraints it is possible that not all feedback modalities will be able to be implemented, but the core modalities (RGB video, audio, haptics) will be delivered.

Risks, Requirements & Constraints

The scope of this project is very large due to the many types of feedback being integrated into a single system, so there is a risk of missing deadlines due to integration issues for more difficult to implement modalities. This could lead to the system not being ready for human testing by the deadline and therefore having no results to present for the project delivery report. To mitigate this, the more achievable core modalities (haptics, audio, RGB video) will be implemented first, with the more difficult modalities to implement (LIDAR, IR) (Kuo et al. (2021)) only being delivered if time constraints allow for it. Additionally, to comply with ethics surrounding human testing, participants are required to be informed on what variables they are being tested and measured on, and surveys collecting personal details will be avoided. Further analysis into risks and ethics regarding this project will be expanded upon in detail in an official risk assessment and in the Human Ethics Low Risk Application to the QUT Research Ethics Committee respectively. The risks of the project in accordance with Appendix B Figure 2 are as follows:

| Hazards Identified | Likelihood of Exposure | Consequences | Control Measures |
| --- | --- | --- | --- |
| Robot in motion – Unexpected robot movement. | Moderate | Extreme – Robot could hit participant or researcher and cause impact injury. | Set an exclusion zone around the robot in a controlled workspace. All testing will be under supervision. Emergency stop button must be implemented, regularly tested for functionality and used if any unexpected movement occurs. |
| Impacting and grasping – Collision with robot and environment. | Moderate | Moderate – Collision could cause moderate damage to equipment. | Set an exclusion zone around the robot in a controlled workspace. Emergency stop button must be implemented, regularly tested for functionality and used if collision is imminent. |
| VR equipment – Participant slips/trips/falls/ becomes disoriented | Moderate | Moderate – Participant may collide with floor or objects while using VR headset. | Set an exclusion zone around the participant in a controlled workspace. All testing will be under supervision. Participant told to stop if they feel disoriented or dizzy. |
| Feedback level – Excessive force or audio feedback | Rare | Extreme/Minor – Excessive haptic feedback could damage fingers and excessive audio feedback could cause discomfort or temporary headaches. | Limit haptic feedback to 5N of force through hardware and software to comply with ISO/TS 15066 (ISO. (2016)). Test all feedback methods before participant use. Emergency stop button must be implemented and used if excessive feedback is detected or reported by participant. |

Quality & Sustainability

The final system’s quality will be determined by the volunteer performance and its perceived usability (SUS), as well as the stability/performance of the system itself. The final system uses QUT’s existing Unitree systems and minimal hardware/resources, and because the G1’s sensors are fully modular any future research or upgrades can be implemented without wasting too many non-renewable resources. Since robots use non-renewable resources in their construction and require energy intensive processes to be made, they need to provide a greater benefit than they cost to make. Since “QUT currently sources 50% of its energy from a new solar farm in Columboola” (Queensland University of Technology. (2023)), Australia’s renewable power percentage for 2026 is 16.67% (Renewable power percentage. (2026)), and “up to 90% of a robot’s lifetime energy footprint is due to operational energy consumption” (PatentPC. (2025)), the final system must provide a benefit greater than 40% of its lifetime energy footprint to be considered sustainable.

Timeline & Deliverables

| Number | Focus | Deliverable/SMART Objective | Dependency | Release Date/ Milestone |
| --- | --- | --- | --- | --- |
| 1 | Literature Review | Literature Review Complete. | None | Week 6 – Project proposal commenced |
| 2 | Project Proposal | Project Proposal Report Complete. | 1 | Week 6 – Project proposal complete |
| 3 | Human Ethics Application & Survey | Draft Human Ethics form & Survey completed and reviewed. Final Human Ethics form submitted. | 2 | Week 7 – Human ethics application complete |
| 3.5 | Risk Assessment | Draft risk assessment completed and reviewed. Final risk assessment submitted. | 2 | Week 7 – Risk Assessment application complete |
| 4 | System Requirements | Specific and measurable system requirements documented. | 2 | Week 8 – Measurable system requirements found |
| 5 | Sub Project 1: Design Haptic Gloves | Design and prototype of haptic feedback gloves. | 4 | Week 8 – Prototype of haptic gloves complete |
| 6 | Sub Project 2: Investigate Individual Feedback | Prototype of individual feedback modes (RGB, audio, haptic). Measurements from the individual feedback modes. | 2, 3.5, 4 | Week 8-Mid Sem Break – Core individual feedback modes implemented |
| 6.5 | Sub Project 2.5: Investigate LIDAR and IR | Prototype of individual feedback modes (LIDAR, IR). Measurements from the individual feedback modes. | 2, 3.5, 4 | Week 8-Mid Sem Break – Extra feedback modes implemented |
| 7 | Detailed design | Refined design of gloves and refined feedback modes. | 4, 5, 6, 6.5 (If possible) | Mid-Sem Break-Week 10 – Detailed design complete |
| 8 | Integrate/ Rapid prototype & experiment | Detailed design implemented. Feedback modes integrated together. | 3.5, 8 | Week 10-13 – Integrated system complete |
| 9 | Interim Progress Report | Interim progress report completed. Interim progress presentation completed. | 1-8 | Week 13 – Interim report of documented work complete. |
| 10 | Bug Fixing/ Pilot | System is functional and able to do testing tasks. | 9 | Week 1-3 – Validated experimental setup |
| 11 | Test & Validate/ Verify | Fully valid testing data from 10-15 participants. | 10 | Week 4-7 – Data collection from 10-15 participants. |
| 12 | Data Analysis | Analysis of testing results. | 11 | Week 4-9 – Results analysed |
| 13 | Draft Project Delivery report/ Presentation | Draft project delivery report and presentation complete. | 12 | Week 10 – Draft project delivery complete |
| 14 | Stakeholder consultation | Demonstration of outcome of project for supervisor. | 13 | Week 10 – Final system demonstration |
| 15 | Project Delivery Report & Oral Defence | Final project delivery report and presentation complete. Project outcome delivered. | 13 | Week 13 – Final project delivery |

Management of Project Changes

Should the project supervisor (Professor Will Browne) request for a change to the scope of the project, specifics on the changes and reasoning for their request can be discussed through emails or during the weekly meetings to make sure it can be implemented correctly. Ideal implementation would minimally impact the final deliverables of the project, with things like small timeline changes and equipment access being easily adjusted for, but completely changing the final deliverables is an action that would require significant re-working of all aspects of the project and thus extensive discussion would be required before anything is implemented.

Sign off

|  | Signature | Date |
| --- | --- | --- |
| Student Engineer |  | 31/08/2026 |

Appendix:

# Appendix A: Review of Literature

## Introduction

Teleoperation, more specifically bilateral teleoperation, is practice of remotely operating a robot while receiving a stream of information feedback from it. Teleoperation effectiveness depends heavily on the quality and quantity of the information feedback a human operator receives, as it allows for the operator to better understand the remote environment and use the robot to accurately manipulate it (Wichmann et al. (2014)). Most teleoperation uses a master-slave system model where the master (human operator) receives monomodal visual data and transmits motion commands to the slave (robot) simultaneously (Wichmann et al. (2014)). Using only visual feedback can give a limited understanding of how the robot is interacting with the environment, with most common situation where visual-only feedback is insufficient being precise manipulation tasks. This means that adding more information in the form of multimodal feedback, specifically haptic and audio, is a very important area of research as “besides visual feedback, tactile feedback to the human operator attempting to accomplish a task can help improve the performance of the system.” (Wichmann et al. (2014)).

### Impact of Haptic Feedback in High Latency Teleoperation for Space Applications (Louca et al. (2024))

This article aimed to analyse the effect that extreme signal latency had on haptic feedback in teleoperation, in the context of teleoperation for space applications. This was investigated by teleoperating a 6-DOF robotic arm to perform line-drawing and peg-in-hole tasks, where a system added delays between the operator input and the feedback from the robot with tests, with delays ranging from 7ms to 2500ms. The tests measured the success rate, completion time, accuracy, contact force and perceived trust and demonstrated trust. For tests with no feedback delay, all haptic feed-back metrics performed better than non-haptics, however for tests with significant delays (>500ms) only contact force had significant improvement, with 68% less force used. This improvement came at the cost of completion time, with haptic feedback being up to 12.1% slower than non-haptics. As delay increased, completion time without haptics mostly plateaued while completion time with haptics kept increasing.

Louca et al. (2024) demonstrates that while haptic feedback is a viable option for mitigating damage to objects being manipulated and can improve teleoperation performance, the effectiveness of the improvement depends on operating conditions like signal latency. As a result, high fidelity haptic feedback will be difficult to implement in bilateral teleoperation as with simultaneous data flow in a single data stream (whether a physical cable or through wireless connection) there is very limited bandwidth available, and high-fidelity feedback information (>20Hz, 1-50ms latency) can take up significant portions of the bandwidth (Gong et al. (2024)). Therefore, only increasing the amount of feedback does not guarantee improved teleoperation performance, as it can introduce latency that decreases performance.

### Embodied robot teleoperation based on high-fidelity visual-haptic simulator: Pipe-fitting example (Zhou et al. (2023))

This article aimed to investigate the effect haptic feedback had on teleoperation performance in the context of a pipe-fitting construction task. It did this by using a high-fidelity simulation of visual-haptic teleoperation feedback for a manipulation task that required significant precision, and tested with metrics such as installation error, deformations to pipes being manipulated and perceived mental load on operator. Results showed that compared to visual-only VR teleoperation, haptic feedback reduced installation errors by 30.5%, deformations to pipes by 37.7% and mental load by 21.5%. Additionally, task time was reduced by 4.6%, showing that similarly to Louca et al. (2024), when the operating conditions did not contain significant delay visual-haptic feedback performed significantly better than visual-only feedback for physical manipulation. One limitation of this article is that the tests only considered the specific pipe-fitting task, so the exact implications of how different teleoperation tasks could benefit from visual-haptic feedback is still unclear, although the results in combination with Louca et al. (2024)’s results suggests a general improvement to performance under ideal circumstances.

### The integration of mobile (tele) robotics and wireless sensor networks: A survey (Wichmann et al. (2014))

This review article aimed to survey teleoperation systems with multimodal feedback, particularly haptic (force), audio and visual (RGB), focusing on communication systems between the robot and the operator, and the flow of feedback information. It found that different modalities had different requirements depending on the system they were implemented in (both physical and computational) and the latency, bandwidth and data throughput of the communication channel between the robot and operator. This is very important for multimodal feedback, particularly haptic force feedback, because time delays and packet loss from bandwidth constraints between input and feedback with force “tends to destabilize a teleoperation system” because “a human operator tends to apply more force when a packet loss is perceived” (Wichmann et al. (2014)). While multimodal feedback in teleoperation helps operators achieve a better understanding of the robot’s environment, the constraints on achieving peak performance are immense and the efficiency of multimodal teleoperation depends on what the communication systems between the operator and the robot can transmit. One limitation of this article is that it focuses on the systems of communication and how feedback information flows instead of doing experimental testing to find how those systems and information feedback affect operator performance and decision making.

## Key themes

The first theme identified is that additional sensory feedback can improve specific aspects of teleoperation performance. Louca et al. (2024) demonstrated that maximum contact force was improved by adding haptic feedback, despite significant signal latency. Zhou et al. (2023) demonstrated that task-time, task-error, damage to objects and mental load were all significantly improved when visual-haptic feedback was introduced. With the findings combined, this suggests that visual-only feedback may not be sufficient for all manipulation tasks, and multimodal feedback may be necessary.

The second theme identified is an inverse of the first, in that adding more feedback does not guarantee improved performance. Louca et al. (2024) demonstrated that haptic feedback was severely affected by large latencies, and while haptic feedback was beneficial for contact forces, it also was a severe detriment for task time. The findings suggest that the successfulness of adding feedback modes to visual-only feedback depend on factors like the quality and latency of the feedback. This implies that additional feedback may increase demands on the operator that can negatively affect performance.

The third theme is that all articles agree there is a benefit to combining multiple feedback modalities. Wichmann et al. (2014) demonstrates how different forms of multimodal feedback can convey different requirements for understanding the robot’s environment, with haptic feedback conveying physical interaction (as demonstrated in Louca et al. (2024) and Zhou et al. (2023)) and audio feedback conveying events or warnings that are outside of the scope for visual feedback (e.g., a human speaking or an occluded object breaking) (Ohno et al. (2026)). Combining these modalities into multimodal feedback could greatly enhance an operator’s understanding of how the robot’s environment and how it interacts with it, however due to the constraints identified in the second theme truly effective multimodal feedback would need to have low latency feedback that is fully synchronised so there is no mismatch between events (Kuo et al. (2021)). Increasing the information feedback may also increase the demands on the operator, as demonstrated in Louca et al. (2024).

An extension of the third theme is that feedback modes not natural to a human’s senses (e.g., LIDAR or IR vision) may also similarly convey important information that regular feedback cannot Wichmann et al. (2014) describes the benefits of various feedback modes and lists LIDAR as a way to “maximize the efficiency of the mobile robots” by conveying depth and special information far more efficiently than RGB vision can. IR sensors could potentially be useful in specific situations, for example, early detection of thermal anomalies in teleoperation for disassembling batteries (Zhao et al. (2024)). Since the effects of haptic and audio feedback were only explored for RGB vision, there could be different affects to performance with different visual modes as haptic feedback enhances an operator’s understanding of the 3D space the robot interacts with, and LIDAR vision enhances an operator’s understanding of the 3D environment of the robot, so there may be a diminished improvement when combining the two modalities. Non-standard feedback combinations is a large research gap that is very relevant to modern teleoperation practices, where situations might require non-standard feedback modes for effective performance (Kuo et al. (2021)).

## Conclusion

In conclusion, current literature indicates that implementing multimodal feedback into teleoperation has the potential to significantly improve operator performance during teleoperation. Haptic feedback can provide useful information for physically interacting with objects (spatial information, object hardness, contact force), however its effectiveness depends on system conditions like signal latency. Experiments using systems with both visual (RGB) and haptic feedback demonstrate significant improvement over visual-only systems for tasks involving manipulation, and research into more varied systems that use other feedback methods (audio, LIDAR, IR) show that using more than visual-haptic feedback can lead to possible performance improvements (Ohno et al. (2026)). However, current literature also indicates that this performance improvement depends on factors like signal latency, connection quality and computational ability, and that the inclusion of extra feedback can negatively affect some aspects of performance, like task completion time (Louca et al. (2024)). Additionally, there was minimal experimental research into both how audio-visual feedback specifically affected teleoperation and how different feedback modalities affect teleoperation with varied visual data. These findings and research gaps have led to the synthesis of the final research question:

“How does the availability of haptic and auditory feedback affect human performance and behaviour during teleoperation of robots with varied visual data?”

# Appendix B: Figures

Figure 1 (Adapted from Childs, N. (2019)) Note. A diagram of the V-shaped project model altered to replace “Pilot release: roll-out” with “Experimental test”

Figure 2 (MOPP. (2024)) Note. A Risk Likelihood-Consequence Scale table used by QUT

# Appendix C: Document Version History

| Version | Date | Author | Changes/Comments |
| --- | --- | --- | --- |
| 1 | 31/08/2026 | Joseph Schneider | Initial project proposal draft |
| 2 | 01/09/2026 | Prof. Will Browne | Feedback on initial project proposal draft |
| 3 | 03/09/2026 | Joseph Schneider | Updated project proposal draft |
| 4 | 03/09/2026 | Prof. Will Browne | Feedback on updated project proposal draft |
| 5 | 04/09/2026 | Joseph Schneider | Final project proposal |

References

Wichmann, A., Okkalioglu, B. D., & Korkmaz, T. (2014). The integration of Mobile (tele) robotics and Wireless Sensor Networks: A survey. Computer Communications, 51, 21–35. https://doi.org/10.1016/j.comcom.2014.06.005

Louca, J., Eder, K., Vrublevskis, J., & Tzemanaki, A. (2024). Impact of haptic feedback in high latency teleoperation for space applications. ACM Transactions on Human-Robot Interaction, 13(2), 1–21. https://doi.org/10.1145/3651993

Zhou, T., Xia, P., Ye, Y., & Du, J. (2023). Embodied robot teleoperation based on high-fidelity visual-haptic simulator: Pipe-fitting example. Journal of Construction Engineering and Management, 149(12). https://doi.org/10.1061/jcemd4.coeng-13916

Ohno, K., Nagano, H., & Yokokohji, Y. (2026). Beyond Visual and force feedback: Role of vibrotactile and auditory cues in robot teleoperated assembly. Robotics, 15(2), 39. https://doi.org/10.3390/robotics15020039

Kuo, C.-Y., Huang, C.-C., Tsai, C.-H., Shi, Y.-S., & Smith, S. (2021). Development of an immersive slam-based VR system for teleoperation of a mobile manipulator in an unknown environment. Computers in Industry, 132, 103502. https://doi.org/10.1016/j.compind.2021.103502

Li, Y., Ye, H., Wu, S., Zhao, X., Liu, Y., Lv, L., Zhang, P., Zhang, X., & Zhou, Y. (2022). Mixed reality and haptic–based dental simulator for Tooth Preparation: Research, development, and preliminary evaluation. JMIR Serious Games, 10(1). https://doi.org/10.2196/30653

Gong, Y., Mat Husin, H., Erol, E., Ortenzi, V., & Kuchenbecker, K. J. (2024). AiroTouch: Enhancing telerobotic assembly through naturalistic haptic feedback of tool vibrations. Frontiers in Robotics and AI, 11. https://doi.org/10.3389/frobt.2024.1355205

Zhang, Z., Lehnert, C., Browne, W., & Roberts, J. (2026). Acoustic feedback for closed-loop force control in robotic grinding. IEEE International Conference on Robotics and Automation (ICRA). https://arxiv.org/html/2602.20596

Shukla, A., & Karki, H. (2016). Application of robotics in offshore oil and gas industry— a review part II. Robotics and Autonomous Systems, 75, 508–524. https://doi.org/10.1016/j.robot.2015.09.013

LucidVR. (2026). LucidVR/lucidgloves: Arduino/ESP32 based DIY VR haptic gloves. compatible with steamvr via OpenGloves. GitHub. https://github.com/LucidVR/lucidgloves

Unitreerobotics. (2026). Unitreerobotics/xr_teleoperate: This repository implements teleoperation of the Unitree humanoid robot using XR devices. GitHub. https://github.com/unitreerobotics/xr_teleoperate

ISO/TS 15066:2016 - robots and robotic devices — Collaborative Robots. (2016). https://www.iso.org/standard/62996.html

Risk management policy. Risk Management Policy / Document / MOPP. (2024). https://www.mopp.qut.edu.au/A/A_02_05.jsp

Childs, N. (2019). Design. Mechanical Design Engineering Handbook, 1–47. https://doi.org/10.1016/b978-0-08-102367-9.00001-9

Zhao, F., Deng, W., & Pham, D. T. (2024). A robotic teleoperation system with integrated augmented reality and Digital Twin Technologies for disassembling end-of-life batteries. Batteries, 10(11), 382. https://doi.org/10.3390/batteries10110382

PatentPC. (2025). Green Robotics: Energy Consumption & Sustainability Stats. PatentPC. https://patentpc.com/blog/green-robotics-energy-consumption-sustainability-stats

Renewable power percentage. Clean Energy Regulator Australian Government crest. (2026). https://cer.gov.au/schemes/renewable-energy-target/renewable-energy-target-liability-and-exemptions/renewable-power-percentage

Queensland University of Technology. (2023) Sustainability. https://www.qut.edu.au/about/priorities/sustainability